IFT 166 Introduction to Internet Networking

Lab 10

Classless Subnetting

Which of the following is the subnet broadcast address for the subnet in which IP address 172.31.77.201/27 resides? (select one)

172.31.201.255

172.31.255.255

172.31.77.223

172.31.77.207

Represent /26 in dotted decimal format (choose one)

255.0.0.0

255.255.255.192

255.255.255.0

255.255.255.128

| What valid host range is the IP address 172.21.149.224/23 a part of? | Between 172.21.148.1 and 172.21.149.254 |
| --- | --- |

| What is the first valid host on the subnetwork that the node 192.168.247.251/26 belongs to? | 172.21.148.1 the network 192.167.247.192/26 |
| --- | --- |

When calculating usable hosts per subnet, the following formula is used 2^bits - 2. For what reason are two addresses subtracted? (Select Two)

Broadcast

Multicast

Unicast

Network

| What is the first valid host on the subnetwork that the node 172.25.130.178/25 belongs to? | 172.25.130.129 |
| --- | --- |

| Find the Broadcast Address of the following Subnet ID: |  |  |  |
| --- | --- | --- | --- |
|  | Subnet ID | Subnet Mask | Broadcast Add |
| 1 | 10.72.0.0 | 255.248.0.0 | 10.79.255.255 |
| 2 | 172.30.64.0 | 255.255.192.0 | 172.31.255.255 |
| 3 | 192.168.6.52 | 255.255.255.252 | 192.168.6.55 |
| 4 | 172.22.54.0 | 255.255.254.0 | 172.22.55.255 |
| 5 | 10.77.3.14 | 255.255.128.0 | 10.77.127.255 |

| How many subnets and hosts per subnet can you get from the network 192.168.98.0 255.255.255.252? | 2 |
| --- | --- |

Number of hosts located on a network, where the IPv4 where subnet make is /27 (choose one)

27

30

32

5

| How many subnets and hosts per subnet can you get from the network 192.168.116.0/28? | 14 |
| --- | --- |

| What is the first valid host on the subnetwork that the node 172.18.232.94 255.255.252.0 belongs to? | 172.18.232.1 |
| --- | --- |

Two ways to represent the network mask that would allow 14 hosts would be:

/14

255.255.255.240

255.255.14.0

/28

| What valid host range is the IP address 192.168.254.181 255.255.255.192 a part of? | from 192.168.254.129 to 192.168.254.190 |
| --- | --- |

| How many subnets and hosts per subnet can you get from the network 192.168.29.0 255.255.255.240? | 14 |
| --- | --- |

| Which subnet does host 172.25.245.73/24 belong to? | 172.25.245.0/24 |
| --- | --- |

| What is the first valid host on the subnetwork that the node 192.168.247.66/29 belongs to? | 192.168.247.65 |
| --- | --- |

| What valid host range is the IP address 172.22.78.202/28 a part of? | between 172.22.78.193 to 172.22.78.206 |
| --- | --- |

| What valid host range is the IP address 192.168.44.139/28 a part of? | 192.168.44.129 to 192.168.44.142 |
| --- | --- |

What are the assignable addresses for the 12th subnet with the network address of 220.100.100.0 and the number of needed subnets is 45? (choose one)

220.100.100.42 to 220.100.100.51

220.100.100.52 to 220.100.100.55

220.100.100.45 to 220.100.100.46

220.100.100.40 to 220.100.100.44

| You have a network address of 200.138.1.0 with a subnet mask of 255.255.255.252. Please list the following: number of networks, number of hosts per network, the full range of the first three networks, and the usable address range from those first three networks. Additionally, identify the broadcast addresses for each network. |  |
| --- | --- |
| Number of networks | 64 |
| Number of hosts | 2 |
| Full range for first three networks: |  |
| 1. | 200.138.1.0 to 200.138.1.3 |
| 2. | 200.138.1.4 to 200.138.1.7 |
| 3. | 200.138.1.8 to 200.138.1.11 |
| Usable range for the first three networks: |  |
| 1. | 200.138.1.1 to 200.138.1.2 |
| 2. | 200.138.1.5 to 200.138.1.6 |
| 3. | 200.138.1.9 to 200.138.1.10 |
| Broadcast addresses for first three networks: |  |
| 1. | 200.138.1.3 |
| 2. | 200.138.1.7 |
| 3 | 200.138.1.11 |