IFT 166 Introduction to Internet Networking

Lab 13

Simple network (routers, switches and computers)

- Objectives: create a simple network with 6 PCs, 2 switches, and a router.

- Identify the proper cable to connect the PCs to the switches. Identify the proper cables to connect a PC and router to each switch. Configure workstation and router IP address information.  Test connectivity using the Ping command.

- Setup the following topology (using only straight-through cable and FastEthernet ports).

- Configure the PCs with the following IP address information

- Include the default gateway as the computers are not directly connected. We essentially have two separate LANs, each connected to its own router.

- On the LAN with Switch0, we will call this the 192.168.1.0 network

- On the LAN with Switch1, we will call this the 192.168.2.0 network

| Computer | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| PC1 | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC2 | 192.168.1.4 | 255.255.255.0 | 192.168.1.1 |
| PC3 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |
| PC4 | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 |
| PC5 | 192.168.2.4 | 255.255.255.0 | 192.168.2.1 |

Completed

- Now will configure the router. Open the router and click the Config tab.

- On this tab, you can change the Display Name or Hostname of the router (if you want).

- Click on the FastEthernet0/0 interface on the left

Completed

- Configure the router with the following IP address information

|  | IP Address | Subnet Mask |
| --- | --- | --- |
| FastEthernet0/0 | 192.168.1.1 | 255.255.255.0 |
| FastEthernet1/0 | 192.168.2.1 | 255.255.255.0 |

- Make sure you tick the port status to be ‘on’

- Carry out the same procedure for FastEthernet1/0 interface

- From PC0 (on the 192.168.1.0 network) ping PC3 (192.168.2.0 network)

- Submit a screenshot of your successful ping in addition to this file.